/* --------------------------------- flight.doc ----------------------------- */

/* This is part of the flight simulator 'fly8'.

* Author: Eyal Lebedinsky (eyal@ise.canberra.edu.au).

*/

The models

----------

Classic

This one simulates a trivial motion model. It always moves in the

direction it is pointing. It has no limits on G loading. It can fly

equaly well underground (so you cannot land it). It is only useful for

learning how the controls work

Basic

A basic simulation which includes some basic features only. It is

more real than the 'classic' but cannot show any of the subtle effects.

Xplane

An experimental model which uses some more common derivatives. It is

now obsolete and probably does not work.

Yplane

A model based on aerodynamic derivates. It is fast (still written in

fixed point). Properly simulates many effects. Reallistic landing gear.

This model will respond to the debug W switch by allowing untrimmed

pitch moment. Normally the only pitch moment is the one from the

elevators (and the corresponding damping).

Fplane

This model is actually two models in one. The first is identical to

Yplane (derivatives) and the second simulates the actual aerodynamics

phenonena. However, this model uses floating point and as such is much

easier to work with.

This model will respond to the debug W switch by allowing untrimmed

pitch moment. Normally the only pitch moment is the one from the

elevators (and the corresponding damping).

If you set the debug Z switch then the fundamental model will be

used, otherwize the derivative based model is used; this is useful for

comparing the two models.

If you set the debug Y switch then the 'yplane' model will be

activated instead; this is useful for comparing the two models (if the Z

option is turned off!).

Adding a new model

------------------

As an example, suppose we want to work on a new model based on the

fundamental model of fplane. Let's call this one the zplane model. We

copy fplane.c to zplane.c and rename the entry point from

dynamics_fplane() to dynamics_zplane(). Now we remove the part which

does the derivative based model and remove the dependency on the debug Y

and W switches.  We now have the new model.

We add the new model to prm.c. This includes the prototype and the entry

in the table initializers list which follows. You must add the new model

at the end of the list.

Now we add the new module to the makefile; just search for one of the

other modules (say 'ofplane') and do the same thing for 'ozplane'.

If we really want to do a clean job then we want to also define a

manifest constant for the new model. To plane.h we add something like

#define MODEL_ZPLANE	5

at the same place where the other models are defined. Note that the

number here is the index into the models table in prm.c. Modifying

plane.h will cause many modules to be recompiled.

Finally, for neatness, we create a new parameter file which we will

modify as the model is developed. In the /parms directory we copy

f16.prc to zplane.prc. We add zplane.prm to the makefile and then (at

the top level) we do 'make parm'. Then we copy the new zplane.prm to the

install directory and change the fly.ini there to default to the new

model using the parameter 'Pzplane'.

Adding a new parameter

----------------------

First we decide on the type (short, fraction, angle or long) and make

a clear comment about it. Also note the units to be used.

We now need to add the new parameter to the following:

plane.h:

This declares the new parameter.

prm.c:

This reads the new parameter. The parameter MUST be added in the

correct position that will be used in the *.prc file (it has nothing to

do with the order in plane.h but we keep everything in the same order

for easier maintenance).

parms/*.prc:

Here we supply the new value. We make sure that the a new parameter

is added to ALL the *.prc files, otherwise they will not be read

correctly.

Finally, use the new parameter in the your model. Note how the program

handles the fixed point data, just follow the exmaples of other

parameters for converting the input values to floating point.

Now, at the top level, we do 'make fly parm install'. Note that if we

changed any files in the install directory then these will be

overwritten, so make sure you keep a copy of these (like the *.ini and

*.vmd and *.max) files elsewhere to be restored.

General notes

-------------

Fly8 uses the following conventions for the axes:

X	right

Y	forward

Z	upward

aX	pitch (up)

aY	roll (clockwise)

aZ	yaw (left)

This is different from the standard aerodynamic conventions where X and

Y are swapped and Z is the oppsite sign. In the fplane model the fly8

values are converted to local variables which conform to the common

standard so one may ignore this problem there.

If you have questions about any other aspect of the program then I can

be contacted as eyal@ise.canberra.edu.au; I will be interested in adding

new material to the program if you care to contribute it.